Control Point/Corner Reporting

Draft Version 2A  5-1-2014

Abstract

This document describes the proposed data content for reporting information on control observations for points (control stations) and corners (land corners) for use in a GIS or database system.  The data content includes added suggested attributes that could be used for a broader control inventory.  This is a proposed reporting data list and does not include all of the information about a control observation that may be needed to replicate an observation.

Significant consideration was given to past experiences with control reporting.  The number of suggested required data elements are minimized to encourage participation.  In cases where the control report is generated through a contract or paid setting, the project specifications could include more extensive data content.  There are also a number of data elements that are “global” to a given project.  This information could be defaulted for each reported record, provided as header information, or provided in a separate related table.  Some information, such as attachments, may have a one-to-many relationship to a control point.

There are two basic categories of control observations.  Control Stations are points of established position used to reference other positions.  Often these are located on National Geodetic Reference Control monuments for horizontal, vertical or three-dimensional control.  Control stations are usually not coincident with corners.  Corners are locations of legal land surveying reference or boundary definition.  Corners may define Public Land Survey System (PLSS), property, subdivision plat, or other legal locations.  Information about the physical marker occupied or measured is typically more important for corners than for stations.  The term Control Point is used throughout this document to refer to either a Control Station or Corner for which a coordinate position is being reported.

Data Categories

The following are the categories of data content.  These categories are provided to simplify the documentation and to group the data content for documentation.

| Category | Description |
| --- | --- |
| Identification | The identification category includes the information to identify, name and locate the control point. It includes items such as the station name, corner identifier, its description, and other identifying labels. |
| Coordinate position | These are coordinate values for the control point. |
| Positional Accuracy (XY and Z) | These are accuracy attributes for the control point. |
| Metadata | The feature metadata about an observation describes the coordinate system, datum, units of measure, and information about the dates and methods of collection. |
| Physical properties | The physical properties describe the marker and field conditions for the control point. This includes the material type, protection, and how to identify the marker in the field. |
| Source | The source describes the data collector or data steward for the reported control point. This includes information such as the surveyor name, agency, project, and contact information. |
| Attachments | The attachments are supporting information such as adjustment reports, monument records, photos, or other clarifying attachments. |

## The following list of data element listings includes the following color coding.

## Proposed Information Model

## The following list of data element listings includes the following color coding.

## Red Mandatory

Orange Mandatory but could be collected as an overall value for a single submission

Green maybe required for clarification

Blue are system assigned fields

| Category | Field | Description | Domain | Why do we need it and Implementation Notes |
| --- | --- | --- | --- | --- |
| Identification | OBJECTID | DBMS\System generated |  | DBMS uniqueness |
| Identification | GUID | Global Unique ID |  | DBMS uniqueness between database systems or for feature linking to related tables as well for check-out\check-in. |
| Identification | Control Point ID | The point’s name. Can be used for labeling. This field should clearly indicate if the control point is on the corner, is a corner offset or accessory, or is control station. Best practice: keep unique for easy updates. |  | An easily readable and understandable identifier the point. Quickly and easily understanding the reported values that are offsets or accessories is very important for use assessment. For western or BLM implementations this could be the GCDB Identifier. |
| Identification | Township/Range/Section | This attribute describes the associated PLSS Township/Range and Section for the control point. |  | To uniquely locate the point within the PLSS Rectangular system and to verify the GCDB point identifier. There are likely to be multiple sections listed and may be multiple Townships listed. These could be broken out into separate fields. |
| Identification | GCDB Identifier | This is the six digit BLM standard corner identifier. |  | For systems using BLM adjustment software and systems this identifier is used for computation and adjustment support. |
| Identification | GCDB Township | This is the 16 digit standard PLSS Township Identifier used to make the GCDB identifier nationally unique. |  | The GCDB PLSS Township number makes the GCDB number nationally unique. |
| Identification | Control Point Type Category | This is the category of the control point. The longer list of all possible corner and station types is simplified into a few categories | Control Station PLSS Rectangular Corner PLSS non- Rectangular Corner Land Survey Corner Corner Accessory Other Unknown | The control point category further identifies the corner or station location and its relationship to existing data in the GIS system. If the identifier does not readily identify the coordinate value as an offset or accessory this field is very important for assessing fitness for use. |
| Identification | Control Point Type | This is the detailed corner type. The domain values are extensive and while this detail adds clarification the long list of types may limit compliance. | (see list at the bottom of page) |  |
| Identification | Control Point Type Note | Special notes to correctly identify the control point such as a witness point, corner accessory or on the corner |  | Mandatory if needed for clarification or to properly locate the intended control point location. |
| Coordinate position | X or Longitude | Field Reported X, East, or Longitude coordinate value |  | This is a reported control coordinate value. This may be broken into separate fields for longitude reporting components for degrees, minutes, and seconds. Longitudes are presumed to be West and West Longitude may be reported as negative. |
| Coordinate position | Y or Latitude | Field Reported Y, North, or Latitude |  | This is a reported control coordinate value. This may be broken into separate fields for longitude reporting components. Latitudes are presumed to be North |
| Coordinate position | Z or Elevation | These are the elevation or height distances. |  | This is a reported control coordinate value. |
| Positional Accuracy | AccuracyXY | Planimetric (horizontal) RMS error in meters. Or cm |  | Used to gather a general indication for the control quality and to assess the control points use in the GIS especially when compared to existing GIS data. |
| Positional Accuracy | AccuracyZ | Vertical RMS error in meters |  | Used to gather a general indication for the control quality and to assess the control points use in the GIS especially when compared to existing GIS data. |
| Positional Accuracy | Accuracy Units | If the planimetric accuracy is reported as a number it may be necessary to report the units of accuracy. |  | This may be necessary to understand the size of the reported accuracy. |
| Metadata | Horizontal Datum | Horizontal datum of the coordinate values | NAD27 NAD83 NAD83 Harn (State) NAD83 (Epoch) Other | This value determines the datum basis for the horizontal coordinate values and is necessary to relate the reported coordinates to the GIS data sets. |
| Metadata | Coordinate Reported Format | This field describes the reported coordinate format. | D-M-S Decimal Degrees Plane Coordinate Other | It is sometimes necessary to understand whether the reported value is in decimal degrees, plane coordinate, or deg-min-sec. if entry fields accommodate the varying formats than this would not be needed. |
| Metadata | Coordinate system and Units | This field describes the plane coordinate system, such as SPC as well as the units. | SPC meters SPC Survey Feet SPC International Feet UTM Meters Local meters Local survey feet Local International Feet Other | This is necessary to understand the coordinate reporting basis for plane coordinate reporting. This information may be broken into two field, one for the coordinate system and one for the units. |
| Metadata | Projection | The projection is only needed for local coordinate systems. The SPC and UTM have defined projections |  | For local or low distortion coordinate projections the GIS will need to have the projection information to convert the reported coordinates to a universal coordinate value. |
| Metadata | Vertical Datum | The vertical datum describes the basis of the elevation measurement |  |  |
| Metadata | Vertical Units | The vertical units provide the measurement units for the elevation or height. | US Feet International Feet Meters Decimal Degrees | The vertical units provide the measurement units for the elevation or height. |
| Metadata | Vertical Basis | This is the type of elevation that is reported | Orthometric Elevation | The elevation basis will help the GIS consume the elevation value and to understand any adjustments that may be needed to bring the elevation in harmony with other elevation products. |
| Metadata | Reported Date | The date assigned to the coordinate value report |  | The reported date is used to determine the currency of the coordinate value and to assess its relationship to existing data |
| Metadata | Observation Date | This is the date of the field observation |  | In some cases, such as when historical information is assembled with current information, it is sometimes valuable to understand the various field observation dates since this may affect expected accuracy. |
| Metadata | Adjustment Date | This is the date of any coordinate adjustment or computation |  | The adjustment date may be used to understand the coordinate value currency and may also be most helpful if historical data is being integrated into the report. |
| Metadata | Collection Method | This is the technology or methods used to capture the coordinate value | GPS Line of Sight (LOS) Digitized GPS-LOS combined Computed Other Unknown | The methods used for the collection can influence the expected accuracy and fitness for use for the coordinate value. For example a digitized point will not have a monument information and while it may be useful reconnaissance will not be located on a field marker. |
| Metadata | Collection Notes | These are any additional notes about the collection method or process such as if the collection is a mix of GPS and line of sight or the source materials if a coordinate value is collected from digitizing |  | The collection notes provide clarification and help understand the fitness for use. |
| Physical | Monument Category | The general category of the marker to indicate the general age of the survey. | Capped Monument Stone or Other Non-Capped Marker Physical feature (Fence Post, Road centerline) Other No Marker Unknown | This field describes the physical field marker category and assists the data user understand the expected application of the point in the GIS as well understanding the fitness for use. |
| Physical | Monument Type | This is the detailed monument type. The list of potential monument types can be quite long and there are always unexpected marker materials. If a project can limit the coordinate recovery to modern monument types, a more limited list of monument types may be manageable |  |  |
| Physical | Access Note | Type of box, locked, subsurface marker in case the top marker gets knocked off Information that helps know how to recover it |  | This information is helpful for marker reconnaissance because understanding access limitations or requirements prior to going to the field can be very helpful |
| Physical | Marker Notes | Provide only one notes field for recovery | More detail on the monument |  |
| Source | Surveyor Name | The name of the surveyor or data steward submitting the data set |  | This may be option but some indication of the submitter, either an agency, firm, or individual needs to be provided. |
| Source | Professional License Number | The licensed land surveyor state license number and needs to include the state |  |  |
| Source | Project Name |  |  |  |
| Source | Contact Telephone |  |  |  |
| Source | Contact Email |  |  |  |
| Source | Agency Name | The agency or organization for the surveyor. In some cases data may be reported from an agency, such as a county, and the land surveyor information may not be provided |  | This information is used to assign the data steward in the standardized data sets |
| Reference | URL |  |  | This is an attachment to the coordinate value report. There may be multiple reference records for a single reported control point. |
| Reference | Photos |  |  |  |
| Reference | Project Documents |  |  |  |

Domain of values for Corner Type

| 1/1024 Section Corner | PLSS | PLSS Aliquot |
| --- | --- | --- |
| 1/256 Section Corner | PLSS | PLSS Aliquot |
| 1/64 Section Corner | PLSS | PLSS Aliquot |
| Aliquot Part Corner | PLSS | PLSS Aliquot |
| Angle Point | Land Survey | Land Survey Point |
| Auxiliary Meander Corner | PLSS | Meander |
| Benchmark | Control | Control Station |
| Block Corner | Land Survey | Land Survey Point |
| Center Quarter Section Corner | PLSS | Quarter Corner |
| Closing Center of Section Corner | PLSS | Quarter Corner |
| Closing Quarter Corner | PLSS | Quarter Corner |
| Closing Section Corner | PLSS | Section Corner |
| Closing Subdivision of Section Corner | PLSS | PLSS Aliquot |
| Closing Township Corner | PLSS | Township Corner |
| Condominium Unit Corner | Land Survey | Land Survey Point |
| Construction Center Line | Land Survey | Land Survey Point |
| Crossing Closing Corner | PLSS | PLSS Aliquot |
| Farm Unit Corner | Land Survey | Land Survey Point |
| Fractional Part of Area Description | PLSS | PLSS Aliquot |
| Government Lot Corner | PLSS | PLSS Aliquot |
| Gravity Station | Control | Control Station |
| Homestead Entry Survey Corner | Land Survey | Land Survey Point |
| Horizontal and Vertical Control Station | Control | Control Station |
| Horizontal Control Station | Control | Control Station |
| Horizontal Station | Control | Control Station |
| Indian Allotment Corner | Land Survey | Land Survey Point |
| Indian Reservation Corner | Land Survey | Land Survey Point |
| International Boundary Monument | International | Land Survey Point |
| International Boundary Reference Mark | International | Land Survey Point |
| Intersection or Center Line | Land Survey | Land Survey Point |
| Intersection Point | Land Survey | Land Survey Point |
| Irrigation Block Corner | Land Survey | Land Survey Point |
| Location Corner | PLSS | Control Station |
| Location Monument | PLSS | Control Station |
| Lot Corner | Land Survey | Land Survey Point |
| Meander Corner | PLSS | Meander |
| Metes and Bounds Description | Land Survey | Land Survey Point |
| Mile Corner or Mile Post | PLSS | Land Survey Point |
| Military Reservation Corner | Land Survey | Land Survey Point |
| Mineral Claim | Land Survey | Land Survey Point |
| Mineral Survey Corner | Land Survey | Land Survey Point |
| Not Applicable | Unknown | Unknown |
| Outlot Corner | Land Survey | Land Survey Point |
| Pit Quantity Baseline | Control | Control Station |
| Point of Intersection | Land Survey | Land Survey Point |
| Point of Tangent | Land Survey | Land Survey Point |
| Point on Curve | Land Survey | Land Survey Point |
| Point on Line | Land Survey | Land Survey Point |
| Property Corner | Land Survey | Land Survey Point |
| Quarter-Section Corner | PLSS | Quarter Corner |
| Reference Monument | PLSS | Control Station |
| Reservation Sixteenth | Land Survey | Land Survey Point |
| Right of Way Center Line | ROW | Land Survey Point |
| Right of Way Corner | ROW | Land Survey Point |
| River Mile | Land Survey | Land Survey Point |
| Section Corner | PLSS | Section Corner |
| Sixteenth Corner | PLSS | PLSS Aliquot |
| Small Holding Claim Corner | Land Survey | Land Survey Point |
| Small Tracts Act Corner | Land Survey | Land Survey Point |
| Special Meander Corner | PLSS | Meander |
| Standard Corner | PLSS | Section Corner |
| Station Point | Control | Control Station |
| Subdivision Boundary Corner | Land Survey | Land Survey Point |
| Subdivision of Section Corner | PLSS | PLSS Aliquot |
| Survey Subdivision Corner | Land Survey | Land Survey Point |
| Tidal Station | Control | Control Station |
| Township Center | PLSS | PLSS Aliquot |
| Township Corner | PLSS | Township Corner |
| Traverse Station | Control | Control Station |
| Unknown | Unknown | Unknown |
| Vertical Control Station | Control | Control Station |
| Witness Center of Section Corner | PLSS | Quarter Corner |
| Witness Monument | PLSS | Control Station |
| Witness Point | PLSS | Control Station |
| Witness Quarter Corner | PLSS | Quarter Corner |
| Witness Subdivision of Section Corner | PLSS | PLSS Aliquot |
| Witness Township Corner | PLSS | Township Corner |